N2: DESIGNING AND SIMULATING CYBER ATTACKS ON DRONES

Encryption Isn’t Enough: Supervised Activity

Inference on Drone Telemetry

Abdulrahim Ünver and Kalle Mossberg

Abstract—This study investigates eavesdropping side channel

attacks on drones using the MAVLink telemetry protocol, by

applying traffic analysis of the payload size and timing intervals

of encrypted communication. By training a supervised model

on data acquired by simulating UAV missions on the PX4

framework, focusing on size and timing related features, different

models capable of detecting events of interest has been produced.

It was found that the strongest model achieved an overall

accuracy of 72% in classification, where some events such as

arming and take-off had perfect accuracy. Size related features

had the biggest importance in classifying events, which implies

that the vulnerability of MAVLink derives from the presence of

distinct payload size. Our work shows that telemetry protocols

with distinct traffic behaviour are vulnerable even if the payload

data is encrypted.

Sammanfattning—Denna rapport undersöker avlyssningsat-

tacker mot drönare som använder telemetriprotokollet MAVLink,

detta genom trafikanalys av payloadstorlek och tidsintervall

av meddelanden i krypterad telemetridata. Genom att träna

en supervised learning-modell på data som samlats in genom

simulering av UAV-uppdrag på PX4-ramverket, detta med fokus

på storlek- och tidsintervallkaraktäristik, har flera modeller

kapabla av att detektera specifik drönaraktivitet av intresse

tränats. Detta har resulterat i att en modell med träffsäkerhet på

72% i klassifiering av händelser skapats, där vissa händelser som

armering och start klassificeras helt korrekt. Storleksrelaterad

karakteristik har visat sig vara av störst vikt vid klassifier-

ing, vilket antyder att sårbarheten hos MAVLink härstammar

från dess distinkta meddelandestorlek. Vårt arbete visar att

telemetriprotokoll med fördefinierad struktur är sårbara även

med krypterad meddelandetrafik.

Index Terms—(MAVLink, AV, Autonomous drone, Traffic anal-

ysis, eavesdropping,AI)

Supervisors: Axel Andersson

TRITA number: TRITA-EECS-EX-2026:145